# Microcontroller-Based Solar Battery Monitoring & Protection System

## 1. Project Overview

This project implements a microcontroller-based monitoring and protection system for a small solar-powered battery and DC load. The system uses a PIC16F877A microcontroller to monitor battery voltage, load current, and solar charging current, while controlling separate relays for the load and charging paths.

The system provides:

- Battery voltage monitoring

- Low-battery load protection

- High-battery indication

- Solar charging control

- Load overcurrent protection

- Automatic retry after an overcurrent event

- Fault-lock mode after repeated failures

- Manual fault reset

- LCD-based monitoring

- Visual and audible status indication

The complete system was designed and tested in Proteus, while the firmware was developed using MPLAB X IDE and XC8.

### Main operating concept

The PIC16F877A continuously measures three analog signals:

- Battery voltage through a voltage divider

- Load current through an ACS712 current sensor

- Solar charging current through a second ACS712 sensor

Based on these measurements, the microcontroller controls the load relay, charging relay, LEDs, buzzer, and LCD.

## 2. System Diagram

The overall system consists of a solar charging path, battery, protected DC load, current sensors, and the PIC16F877A-based monitoring and control unit.

SOLAR CHARGING PATH

│

18 V Solar Source

│

1N4007

│

10 Ω

│

Solar ACS712 Sensor

│

Charging Relay

│

▼

┌─────────────┐

│   BATTERY   │

└─────────────┘

│         │

Voltage   │         │ Load Supply

Divider  │         │

│         ▼

AN0      Load ACS712

│         │

│         ▼

│     Load Relay

│         │

│         ▼

│      DC Motor

│

┌──────▼──────────────┐

│     PIC16F877A      │

│                     │

│  ADC Monitoring     │

│  Protection Logic   │

│  Relay Control      │

│  LCD Control        │

│  Status Indication  │

└──────┬──────────────┘

│

┌─────────────┼─────────────┐

│             │             │

LCD          LEDs          Buzzer

│

Reset Button

The PIC16F877A acts as the central controller. It receives the three sensor signals through its ADC channels and controls the load relay and charging relay according to predefined protection conditions.

## 3. Hardware & Pin Configuration

### Main Components

| Component | Function |
| --- | --- |
| PIC16F877A | Main microcontroller |
| LM016L 16×2 LCD | Displays system measurements and status |
| ACS712-05B ×2 | Load and solar charging current measurement |
| 12 V Relay ×2 | Load and charging path control |
| BC547 ×2 | Relay driver circuits |
| 1N4007 diodes | Relay flyback and solar-path diode |
| 30 kΩ + 10 kΩ | Battery voltage divider |
| 10 Ω resistor | Limits simulated solar charging current |
| DC motor | Simulated electrical load |
| 18 V DC source | Simulated solar panel |
| LEDs | Battery status indication |
| Buzzer | Protection warning |
| Push button | Manual fault reset |
| 4 MHz crystal + 22 pF capacitors | Microcontroller clock |

### PIC16F877A Pin Configuration

| PIC Pin | Signal | Function |
| --- | --- | --- |
| RA0 / AN0 | Battery voltage | Measures battery voltage |
| RA1 / AN1 | Load ACS712 | Measures load current |
| RA2 / AN2 | Unused | Reserved |
| RA3 / AN3 | Solar ACS712 | Measures charging current |
| RB0 | LOW LED | Indicates low battery |
| RB1 | LCD RS | LCD register select |
| RB2 | LCD EN | LCD enable |
| RB3 | NORMAL LED | Indicates normal battery condition |
| RB4 | LCD D4 | LCD data |
| RB5 | LCD D5 | LCD data |
| RB6 | LCD D6 | LCD data |
| RB7 | LCD D7 | LCD data |
| RC0 | Load relay | Controls DC load |
| RC1 | HIGH LED | Indicates high battery voltage |
| RC2 | Buzzer | Protection warning |
| RC3 | Charging relay | Controls solar charging |
| RD0 | Reset button | Manually resets fault-lock state |

### Important Connections

Battery voltage measurement:

Battery +

│

30 kΩ

│

├──────── RA0 / AN0

│

10 kΩ

│

GND

Load relay driver:

RC0 ── 1 kΩ ──► BC547

│

└── Relay coil ── +12 V

A 1N4007 flyback diode is connected across the relay coil, with its cathode connected to +12 V.

Charging relay: RC3 controls a second BC547 driver and relay in the same way.

The two ACS712 sensors are connected in series with their respective current paths, while their analog outputs are connected to AN1 and AN3.

## 4. Mathematical Calculations

### 4.1 Battery Voltage Measurement

The battery voltage is reduced using a voltage divider consisting of 30 kΩ and 10 kΩ resistors before being connected to the PIC16F877A ADC.

The ADC input voltage is:

V_ADC = V_BAT × 10 kΩ / (30 kΩ + 10 kΩ)

Therefore,

V_ADC = 0.25 × V_BAT

For example, for a 12 V battery:

V_ADC = 12 × 0.25 = 3 V

Since the PIC16F877A uses a 10-bit ADC with a 5 V reference, the ADC input voltage is calculated as:

V_ADC = (ADC_value × 5000) / 1023 mV

Therefore, the actual battery voltage is:

V_BAT = (ADC_value × 20000) / 1023 mV

This conversion is implemented in the firmware to obtain the battery voltage in millivolts.

### 4.2 ACS712 Current Measurement

The ACS712-05B provides an approximately 2.5 V output at zero current and has a sensitivity of approximately 185 mV/A.

First, the ADC value is converted into sensor output voltage:

V_sensor = (ADC_value × 5000) / 1023 mV

The measured current is then calculated as:

I = (V_sensor − 2500) / 185 A

In the firmware, the calculation is performed in milliamperes:

I_mA = ((V_sensor − 2500) × 1000) / 185

A small zero-current deadband is also used to prevent tiny ADC and sensor offsets from appearing as a false current reading.

### 4.3 Expected Solar Charging Current

The simulated solar source is 18 V. A 1N4007 diode and a 10 Ω resistor are included in the charging path.

Assuming an approximately 0.7 V diode drop and a 12 V battery:

I = (18 − 0.7 − 12) / 10

I ≈ 0.53 A

The simulated system produced approximately 0.51–0.54 A, which is consistent with the expected value considering the sensor model, ADC resolution, and simulation conditions.

### 4.4 Relay Coil Current

The relay coil has a resistance of approximately 240 Ω and is operated from a 12 V supply.

Using Ohm's law:

I_relay = V / R

I_relay = 12 / 240 = 0.05 A

Therefore,

I_relay ≈ 50 mA

Because the PIC16F877A should not directly drive the relay coil, a BC547 transistor driver is used between the microcontroller output and the relay coil.

### 4.5 Protection Thresholds

The firmware uses the following thresholds:

| Parameter | Value |
| --- | --- |
| Low-battery limit | 10.5 V |
| High-battery indication | 14.4 V |
| Charging stop voltage | 14.4 V |
| Charging resume voltage | 13.8 V |
| Load overcurrent limit | 1.5 A |
| Overcurrent cooldown | 3 s |
| Maximum automatic retries | 3 |

The charging system uses hysteresis. Charging stops when the battery voltage reaches or exceeds 14.4 V, but it does not resume until the battery voltage falls to or below 13.8 V. This prevents the charging relay from repeatedly switching on and off around a single threshold.

These voltage limits are simulation/design values for this project and should not be considered universal charging specifications for every battery chemistry.

## 5. Software and Code Explanation

The firmware was developed in MPLAB X IDE v6.35 using the XC8 v4.00 compiler for the PIC16F877A microcontroller. The program continuously reads the analog sensors, evaluates the battery and load conditions, controls the relays, and updates the LCD and protection indicators.

### 5.1 Program Structure

The main program follows this sequence:

- Initialize the microcontroller I/O ports.

- Initialize the ADC.

- Initialize the LCD.

- Enable the load and charging systems.

- Read battery voltage.

- Read load current.

- Read solar charging current.

- Check battery protection conditions.

- Control the solar charging relay.

- Execute the load overcurrent protection state machine.

- Update the LEDs, buzzer, and LCD.

- Repeat continuously.

The system therefore operates continuously without requiring user intervention during normal operation.

### 5.2 ADC Configuration and Sensor Reading

Three analog channels are used:

- AN0 (RA0): Battery voltage

- AN1 (RA1): Load current

- AN3 (RA3): Solar charging current

The PIC16F877A's 10-bit ADC converts each analog signal into a digital value from 0 to 1023. The firmware then converts these ADC values into meaningful electrical quantities using the equations described in Section 4.

The ADC reading process consists of selecting the required channel, allowing a short acquisition time, starting the conversion, waiting for completion, and combining the high and low ADC result registers.

### 5.3 Battery Status and Load Protection

The battery voltage is compared with two main limits.

Low battery:

If the battery voltage falls below 10.5 V, the system:

- Turns ON the LOW LED.

- Turns OFF the load relay.

- Prevents the load from operating until the battery returns to the normal range.

Normal battery:

When the battery voltage is between the defined limits, the NORMAL LED is turned ON and the load is allowed to operate unless another protection condition is active.

High battery:

When the battery voltage exceeds 14.4 V, the HIGH LED is turned ON.

The high-battery indication and the charging cutoff are separate functions. The HIGH LED indicates the battery condition, while the charging relay is controlled by the dedicated charging-control logic.

### 5.4 Solar Charging Control

The charging relay is controlled through RC3.

A hysteresis-based control method is used:

- Battery voltage ≥ 14.4 V → charging OFF

- Battery voltage ≤ 13.8 V → charging ON

The variable charging_enabled stores the current charging state.

This prevents rapid relay switching when the battery voltage fluctuates around the charging cutoff voltage.

### 5.5 Load Overcurrent Protection

The load current is continuously monitored through the ACS712 sensor connected to AN1.

The overcurrent threshold is set to:

1.5 A

When the measured current reaches or exceeds this value, the load relay is immediately switched OFF.

The system then enters a protection sequence consisting of:

NORMAL → COOLDOWN → RETRY → NORMAL

If the overcurrent condition continues, the system repeats the retry process. After three unsuccessful retries, the system enters:

FAULT LOCK

In the fault-lock state, the load relay remains OFF and automatic restarting is disabled.

### 5.6 Cooldown and Retry

After an overcurrent event, the system waits for approximately 3 seconds before attempting to reconnect the load.

The purpose of the cooldown period is to allow a temporary fault or excessive current condition to disappear before attempting another startup.

During a retry:

- The load relay is energized.

- The system waits briefly for the load current to stabilize.

- The current is measured again.

- If the current is below the threshold, normal operation resumes.

- If the current remains above the threshold, the relay is disconnected again and another cooldown cycle begins.

After three failed attempts, the system enters the fault-lock state.

### 5.7 Manual Fault Reset

A push button connected to RD0 is used to reset the fault-lock condition.

The button uses an active-low configuration:

- Button released → RD0 = HIGH

- Button pressed → RD0 = LOW

When the system is in FAULT LOCK and the button is pressed, the retry counter is cleared and the protection state returns to NORMAL.

A short software delay is used to avoid unintended operation caused by mechanical switch bouncing.

### 5.8 LCD Display

A 16×2 LCD is connected to PORTB in 4-bit mode.

The display alternates between two information screens:

Battery and load screen

- Battery voltage

- Load current

Solar and charging screen

- Solar charging current

- Charging status

Each screen remains visible for approximately 2 seconds.

Protection messages such as OVERCURRENT, COOLDOWN, and FAULT LOCK take priority over the normal measurement screens so that important fault information remains visible.

### 5.9 Buzzer and Status Indicators

The system provides both visual and audible protection indications.

The LEDs indicate:

- LOW LED: Battery voltage below 10.5 V

- NORMAL LED: Battery voltage in the normal range

- HIGH LED: Battery voltage above 14.4 V

The buzzer is controlled through RC2. It remains OFF during normal operation and toggles during protection states, producing an intermittent warning sound.

### 5.10 Overall Control Logic

The complete control logic can be summarized as:

START

│

▼

Initialize MCU

│

▼

Read ADC Sensors

│

┌─────────┼─────────┐

│         │         │

▼         ▼         ▼

Battery    Load      Solar

Voltage   Current    Current

│         │         │

▼         ▼         ▼

Battery      Check    Charging

Status    Overcurrent  Control

│         │         │

└─────────┼─────────┘

│

▼

Update LCD/LED

│

▼

Update Buzzer

│

▼

Repeat Loop

The overcurrent protection state machine operates within this continuous control loop and can override normal load operation whenever a fault is detected.

### 5.11 Complete Source Code

The following is the complete firmware developed for the PIC16F877A. The program integrates ADC-based sensing, battery monitoring, solar charging control, load protection, overcurrent detection, automatic retry, fault locking, LCD display, status LEDs, buzzer control, and manual fault reset.

The code was developed and tested using MPLAB X IDE v6.35 and XC8 v4.00 with a 4 MHz crystal oscillator.

The major software modules are:

- LCD functions: Initialize and control the 16×2 LCD in 4-bit mode.

- ADC functions: Read battery voltage, load current, and solar charging current.

- Battery voltage conversion: Convert the ADC reading into battery voltage in millivolts.

- Current conversion: Convert ACS712 output into current in milliamperes.

- Battery protection: Detect low, normal, and high battery conditions.

- Charging control: Control the charging relay using 14.4 V stop and 13.8 V resume thresholds.

- Overcurrent protection: Disconnect the load when current reaches 1.5 A.

- Retry mechanism: Automatically attempt to reconnect the load up to three times.

- Fault lock: Permanently disconnect the load after repeated overcurrent failures until manually reset.

- LCD display: Alternate between battery/load information and solar/charging information.

- Buzzer control: Provide an intermittent warning during protection states.

The complete source code is included below.

Note: The source code is maintained as a separate .c file in the project files. The Proteus schematic and MPLAB X project are also included with the project so that the simulation can be opened and reproduced directly.

## 6. Testing and Results

The complete system was simulated and tested in Proteus under different battery, charging, and load conditions. The main protection and control functions were verified individually and as part of the complete system.

### 6.1 Test Results

| Test Condition | Expected Behavior | Observed Result |
| --- | --- | --- |
| Battery = 12 V | Normal operation, load enabled, charging enabled | System operated normally |
| Battery = 10 V | Load disconnected and LOW LED activated | Verified |
| Battery = 14.5 V | Charging disconnected and HIGH LED activated | Verified |
| Battery = 12 V with solar source connected | Solar charging current detected | Approximately 0.54 A |
| Normal motor operation | Load current measured | Approximately 1.06 A |
| Load current ≥ 1.5 A | Load relay disconnected | Verified |
| Overcurrent persists | System enters cooldown and attempts automatic retry | Verified |
| Three unsuccessful retries | Load remains disconnected in FAULT LOCK | Verified |
| RD0 reset pressed during FAULT LOCK | Fault cleared and normal operation restored | Verified |
| Charging cutoff reached | Charging relay disconnected | Verified |
| Battery falls to 13.8 V or below | Charging resumes | Verified |

### 6.2 Normal Operation

At approximately 12 V battery voltage, the system operated normally. The load relay remained energized, allowing the DC motor to operate, while the solar charging path remained active.

The measured load current was approximately 1.06 A, while the solar charging current was approximately 0.54 A.

The LCD displayed the battery voltage, load current, solar current, and charging status through its rotating display screens.

### 6.3 Low-Battery Protection

The battery voltage was reduced to approximately 10 V to test the low-battery protection.

When the voltage fell below the 10.5 V threshold, the LOW LED was activated and the load relay was switched OFF. This prevented the motor from continuing to draw power from the low-voltage battery.

### 6.4 Solar Charging Cutoff

The battery voltage was increased to approximately 14.5 V.

The charging-control logic detected that the battery had exceeded the 14.4 V charging-stop threshold and switched OFF the charging relay. As a result, the measured solar charging current dropped to approximately 0 A.

The HIGH LED was also activated because the battery voltage exceeded the high-voltage indication threshold.

### 6.5 Overcurrent Protection

An additional 10 Ω load resistance was used in parallel with the motor to increase the load current and simulate an overcurrent condition.

When the measured load current exceeded the 1.5 A protection threshold, the load relay was immediately disconnected.

The system then entered the cooldown state, waited approximately 3 seconds, and attempted to reconnect the load.

### 6.6 Automatic Retry and Fault Lock

The overcurrent condition was intentionally maintained to test repeated failures.

The system attempted to reconnect the load up to three times. When all three attempts failed because the overcurrent condition remained present, the system entered FAULT LOCK.

In this state, the load relay remained OFF and automatic restarting was disabled.

The LCD displayed the fault condition and the buzzer provided an audible warning.

### 6.7 Manual Fault Reset

The push button connected to RD0 was pressed while the system was in FAULT LOCK.

The system detected the active-low button signal, cleared the retry counter, returned the protection state to NORMAL, and allowed the load to operate again when the battery and load conditions were within the permitted range.

### 6.8 Overall Result

All major functions of the simulated system were successfully verified, including:

- Battery voltage measurement

- Load current measurement

- Solar charging current measurement

- Low-battery load protection

- High-battery indication

- Automatic charging cutoff and resume

- Load overcurrent protection

- Automatic retry

- Fault-lock operation

- Manual fault reset

- LCD monitoring

- LED status indication

- Audible protection warning

The successful simulation confirms that the developed firmware and control logic perform the intended monitoring and protection functions under the tested conditions.

## 7. Limitations and Notes

The project was developed and validated as a Proteus-based simulation. Therefore, the results represent simulated system behavior rather than measurements from a physical hardware prototype.

The following points should be considered:

- The 18 V DC source is used as a simplified representation of a solar panel.

- The system does not implement MPPT or a dedicated battery charging algorithm.

- The 10 Ω resistor in the charging path is used to limit the simulated charging current.

- The ACS712 and other sensor models in Proteus may produce slightly different readings from real hardware.

- The battery voltage thresholds used in the project are design/simulation values and would need to be selected according to the actual battery chemistry and charging requirements in a hardware implementation.

- The relay control uses transistor drivers and flyback protection, but a practical hardware implementation would require appropriate PCB design, component ratings, wiring, and additional electrical safety considerations.

- The overcurrent protection uses blocking software delays during the cooldown and retry sequence. This is sufficient for the current project but could be improved using timer-based or interrupt-based control in a more advanced implementation.

## 8. Conclusion

A PIC16F877A-based solar battery monitoring and protection system was successfully designed and simulated in Proteus. The system integrates battery voltage monitoring, load and charging current measurement, solar charging control, low- and high-voltage indication, overcurrent protection, automatic retry, fault locking, and manual fault reset.

The project demonstrates the practical use of microcontroller ADCs, voltage dividers, ACS712 current sensors, transistor-driven relays, LCD interfacing, protection logic, and embedded C programming in a renewable-energy application.

The complete MPLAB X source code and Proteus project files are maintained separately so that the system can be inspected, modified, and reproduced.